|  | ## AS 350 B3e 360˚ PRE-FLIGHT INSPECTION |  |
| --- | --- | --- |

RP-C ________________								           Date _______________

STATION 1										    Status	    Initial

1. Transparent Panels			-	Condition - Cleanliness			    __________	    __________

2. MGB - Engine oil cooler air inlet	-	Check no obstruction nor debris		    __________	    __________

3. Side slip indicator			-	Condition				    __________	    __________

4. Pitot tube				-	Cover removed - Condition		    __________	    __________

5. Landing lights			-	Condition				    __________	    __________

STATION 2

1. Front door				-	Condition jettison system check		    __________	    __________

2. Rear door				-	Condition, closed or open locked          											(sliding door)				    __________	    __________

3. Left cargo door			-	Open					    __________	    __________

4. Loads and objects carried		-	Secured				    __________	    __________

5. Left cargo door			-	Closed, locked				    __________	    __________

6. Fuel tank and system			-	Filler plug closed - Tank sump drained	    __________	    __________

7. MGB cowl				-	MGB oil level - Cowl locked		    __________	    __________

8. All lower fairings panels		-	Locked					    __________	    __________

9. Landing gear and footstep		-	Secure - Visual Check			    __________	    __________

10. Static ports				-	Clear, covers removed			    __________	    __________

11. OAT sensor, antennas		-	Condition				    __________	    __________

12. Main rotor head blades		-	Visual inspection, no impact		    __________	    __________

13. Engine air intake			-	Clear (water, snow foreign object)	    __________	    __________

14. Engine cowl				-	Locked					    __________	    __________

15. Exhaust cover			-	Removed				    __________	    __________

16. Rear cargo door			-	Open					    __________	    __________

17. Loads and object carried		-	Secured				    __________	    __________

18. ELT					-	Check ARMED				    __________	    __________

19. Rear cargo door			-	Closed, locked				    __________	    __________

20. Oil drain				-	No oil under scupper			    __________	    __________

STATION 3

1. Heat shield on tail drive		-	Condition, attachment			    __________	    __________

2. Tail boom, antennas			-	Condition - Fairings fasteners locked	    __________	    __________

3. Stabilizer, fin, external lights		-	General condition			    __________	    __________

4. Tail rotor guard (if fitted)		-	Condition, attachment			    __________	    __________

5. TGB fairing				-	Secured, fasteners locked		    __________	    __________

6. TGB oil level				-	Checked				    __________	    __________

7. Tail skid				-	Condition, attachment			    __________	    __________

8. Flexible Coupling			-	Visual Check No Crack			    __________      __________

STATION 4

1. Tail rotor head			-	Condition, laminated bearing. Check for 											separation, cracks, etc.			    __________	    __________

2. Tail rotor blades			-	Visual inspection, no impact		    __________	    __________

3. Stabilizer, fin, external lights		-	General condition			    __________	    __________

4. Tail boom, antennas			-	Condition - Fairings fasteners locked	    __________	    __________

5. Heat shield on tail drive		-	Condition, attachment			    __________	    __________

STATION 5

1. Oil drain				-	No oil under scupper			    __________	    __________

2. EPU door				-	Closed or EPU plugged-in		    __________	    __________

3. Engine cowl				-	Locked					    __________	    __________

4. Right cargo door			-	Open					    __________	    __________

5. Loads and objects carried		-	Secured				    __________	    __________

6. Right cargo door			-	Locked					    __________	    __________

7. Main rotor head and blades		-	Visual inspection, no impact		    __________	    __________

8. MGB cowl				-	No foreign object on transmission deck.											Cowl locked				    __________	    __________

9. Hydraulic oil level			-	Check reservoir level			    __________	    __________

10. Engine oil level			-	Check reservoir level			    __________	    __________

11. Landing gear and footstep		-	Secure - Visual check			    __________	    __________

12. All lower fairing panels		-	Locked					    __________	    __________

13. Door				-	Condition, jettison system check		    __________	    __________

14. External mirror (if fitted)		-	Set to avoid dazzling (night flight)	    __________	    __________

INTERIOR CHECK

1. Cabin				-	Clean					    __________	    __________

2. Fire extinguisher			-	Secured - Check			    __________	    __________

3. Fuses or breakers			-	All set					    __________	    __________

4. Loads and objects carried		-	Stowed and secured			    __________	    __________

5. Front doors jettison system		-	Check - Plastic guard condition		    __________	    __________

TURN AROUND CHECK

1. Overall aspect			-	Condition, cleanliness			    __________	    __________

2. Engine / MGB / TGB			-	Oil level					    __________	    __________

3. Main and tail rotor blades (ground)	-	Condition				    __________	    __________

4. Loads				-	Secured				    __________	    __________

5. All cowlings				-	Locked					    __________	    __________

6. Doors				-	Closed or open-locked (sliding door)	    __________	    __________

FLIR CAMERA

1.Boom					-	Security- General condition		    __________     __________

2.Camera				-	Condition, attachment points		    __________     __________

SLING

1.Sling					-	Security- General condition		    __________     __________

2.Cable and Pins 			-	Condition, attachment points		    __________     __________

FLOATS

1.LH & RH Floats			-	Security- General condition 		    __________     ___________

2.Cylinder				-	Pressure & Condition, attachment points   ___________    ___________

3.Hoses				-	Condition, attachment points		   ___________    ___________

MANDATORY ONBOARD

1.First Aid Kit				-	Condition, no expired			   ___________    ___________

2.Life Vest				-	Condition, cleanliness & no damage	   ___________    ___________

3.Life-raft				-	Condition, cleanliness & no damage	   ___________    ___________

4.AXL					-	Security- General condition		   ___________    ___________

5.Fire Extinguisher			-	Security- General condition		   ___________    ___________

6.Certificate of Airworthiness		-	Onboard				   ___________    ___________

7.Certificate of Registration		-	Onboard				   ___________    ___________

8.Radio License				-	Onboard				   ___________    ___________

9.Flight Logbook			-	Onboard				   ___________    ___________

F.O.B __________________

Released by:									  Accepted by:

_______________________________						  _______________________________

Mechanic 									    Pilot

_______________________________						  _______________________________

A & P License Nr.									  CHPL Nr.