|  | ## AS 350 B3e POST FLIGHT INSPECTION |  |
| --- | --- | --- |

RP-C __________					       	 	                                                                 Date _______________

STATION 1										    Status	                    Initial

1. Transparent Panels			-	Condition, no cracks, cleanliness,												clean if necessary				    __________	    __________

2. Doors pillars				-	Condition, no crack				    __________	    __________

3. Pitot tube				-	Condition, blanking cap removed or fitted											as necessary				    __________	    __________

4. Sideslip indicator				-	Condition					    __________	    __________

5. MGB – Engine oil cooler air inlet		-	Condition, no obstruction nor debris,												blanking removed or fitted as necessary		    __________	    __________

STATION 2

1. Front door jettison system			-	Condition, no crack on external jettison lever	    __________	    __________

2. Left cabin access doors			-	Condition, security, locking, no abnormal freeplay	    __________	    __________

3. Landing gear				-	Condition of crosstubes, skids, wear resistant 											plates, footstep attachment 		  	    __________	    __________

4. Static pressure points			-	Condition, blanking removed or fitted as	necessary    __________	    __________

5. OAT probe				-	Condition, attachment			    __________	    __________

6. Antennas under belly			-	Condition					    __________	    __________

7. Landing and taxiing lights			-	Condition					    __________	    __________

8. Lower cowlings				-	Condition, security				    __________	    __________

9. Left cargo door				-	Opening, condition, attachment points,												no abnormal freeplay			    __________	    __________

10. Left cargo door				-	Closed and secured				    __________	    __________

11. Fuel tank				-	Filler plug closed – Tank sump drained												(before first flight of the day and any 												aircraft displacement			    __________	    __________

12. Rear cargo door				-	Opening, Condition, attachment points, 												no abnormal freeplay			    __________	    __________

13. Rear cargo bay				-	Harness condition				    __________	    __________

14. E.L.T					-	Condition, security, “ARM” or “OFF” as necessary	    __________	    __________

15. Rear cargo door				-	Closed and secured				    __________	    __________

16. LH side MGB and engine cowlings		-	Opening, condition of locking devices, 												no abnormal freeplay			    __________	    __________

17. Upper cowling				-	Security					    __________	    __________

18. MGB					-	Condition, oil level, no leaks			    __________	    __________

19. Transmission deck			-	Cleanliness				    __________	    __________

20. MGB support bars			-	Condition, security				    __________	    __________

21. Hydraulic system			-	Condition, attachment points, pipes, no leaks	    __________	    __________

22. Servos				-	Security, no leaks or cracks			    __________	    __________

23. Cooling fan				-	Motor security, blades condition		    __________	    __________

24. Gimbal ring assembly			-	Fitting, safety pin set and locked		    __________	    __________

25. Electrical harnesses			-	Condition, security				    __________	    __________

26. Fuel shut-off valve			-	Condition, security, no interference		    __________	    __________

27. MGB cowling (LH side)			-	Closed and secured				    __________	    __________

ENGINE AND ENGINE BAY

1. Engine air inlet				-	Security, condition, seal condition		    __________	    __________

2. Firewall				-	Condition, check for cracks			    __________	    __________

3. Engine and accessories			-	General condition, cleanliness, sealing,												attachment, pipes, electrical harness		    __________	    __________

4. Engine transmission deck			-	Condition, cleanliness, no leak		    __________	    __________

5. Engine case				-	Mounting pads condition			    __________	    __________

6. Oil filter				-	Clogging indicator retracted			    __________	    __________

7. Fuel filter				-	Clogging indicator retracted			    __________	    __________

8. Oil system				-	Check for leaks				    __________	    __________

9. Engine mounts				-	Condition, security				    __________	    __________

10. Engine deck drain holes			-	Free from obstructions and debris		    __________	    __________

11. Exhaust pipe				-	Condition, blanking fitted or removed, as necessary   __________	    __________

STATION 3

1. Tail boom				-	Condition, condition of antennas		    __________	    __________

2. Transmission fairings			-	Security, heat shield condition			    __________	    __________

3. TGB fairing				-	Condition, damage or cracks			    __________	    __________

4. Horizontal stabilizer, fin			-	Condition, attachment points, condition 												of external lights and fin antennas		    __________	    __________

5. Tail rotor guard (if fitted)			-	Condition, security				    __________	    __________

6. Tail guard				-	Condition, attachment			    __________	    __________

STATION 4

1. TGB					-	Condition, attachment, oil level		    __________	    __________

- Bellcrank hinge pin			-	No freeplay				    __________	    __________

- Sealant bead			-	Condition					    __________	    __________

- Horizontal Stabilizer			-	Condition, attachment points, external lights										condition

2. Tail rotor blades				-	Attachment, condition of polyurethane												protection coating in the trailing edge 												(visual inspection for debonding, blisters, 											scratches, cracks, dents and distortion). 												On the stainless steal leading edge, 												inspect for impacts, holes (erosion), 												spaying and dents.				    __________	    __________

3. Tail rotor blade spar			-	Check for abnormal spar noise when the rotor is											bent inwards and outwards to form an arc	    __________	    __________

- Laminated half-bearing		-	No bonding separation, deep crack or emergence	    __________	    __________

- Blade horn			-	No play					    __________	    __________

4. Tail rotor head				-	Condition, security			 	    __________	    __________

- Pitch change rod

- Paint line on pitch change control and spacer/baffle					    __________	    __________

- Pitch change rod swivel bearing							    __________	    __________

- Absence of freeplay by twisting the blades back and forth, low amplitude movements		    __________	    __________

- Condition of ball joint (visual inspection)							    __________	    __________

- No Teflon materials has squeezed out							    __________	    __________

- No signs of burnishing or scoring							    __________	    __________

5. Balance arm hinge			-						    __________	    __________

- Cup on either side of the pin								    __________	    __________

- Flapping bearing									    __________	    __________

- Bearing outside rubber cone								    __________	    __________

6. Freewheel				-	Check: operate from tail rotor, free turbine is driven										when tail rotor is manually driven clockwise, the 											freewheel should de-synchronize when tail rotor is											driven anti-clockwise			    __________	    __________

7. Tailboom				-	Condition, condition of antennas		    __________	    __________

8. Transmission fairings			-	Security, attachment (particularly the 6 securing 											points on the fairing, use a mirror if necessary	    __________	    __________

STATION 5

ENGINE AND ENGINE BAY

1. Engine air inlet				-	Security, condition, seal condition		    __________	    __________

2. Firewall				-	Condition, check for cracks			    __________	    __________

3. Engine and accessories			-	General condition, cleanliness, sealing, 												attachment, pipes, electrical harness		    __________	    __________

4. Engine transmission deck			-	Condition, cleanliness, no leak		    __________	    __________

5. Engine case				-	Mounting pads condition			    __________	    __________

6. Oil system				-	Check for list				    __________	    __________

7. Engine magnetic plugs			-	No chip on aft and forward plugs 												(without electrical indication) 			    __________	    __________

8. Engine mounts				-	Condition, security				    __________	    __________

9. Engine deck drain holes			-	Free from obstructions and debris		    __________	    __________

10. Engine cowling				-	Closed and secured				    __________	    __________

MAIN ROTOR SHAFT

1. Swashplate bearing			-	No abnormal hand-felt heating, no grease											runs, no color change or paint scaling	 	   __________	    __________

NOTE: CHECK TO BE PERFORMED WITHIN FIVE MINUTES AFTER ROTOR STOP.

1. Scissors, swashplates, rods swivel bearings	-	Condition, security, freeplay evolution												(manual check)				   __________	    __________

2. Swash plate/pitch change rods end-fittings 											 3. interface				-	No contact traces or paint scaling on swashplate											driving yokes.				   __________	    __________

3. Pitch change rods			-	Condition, no radial free play at end fittings, paint											marks visible and aligned			   __________	    __________

4. Rotor shaft, all visible parts, particularly under										               5. the hub					-	Paint condition, no cracks, crazing, blistering,											corrosion nor tools marks			   __________	    __________

MAIN ROTOR HEAD

1. Main Rotor Head				-	Security, general condition			   __________	    __________

2. STARFLEX star				-	no delamination, (splinters)			   __________	    __________

3. Star recesses				-	No cracks

4. Spherical thrust bearings frequency adapters	-	No elastomeric defects, separation, scratches,											blisters, extrusion or cracks (other than minor and											non evolving surface defects)			   __________	    __________

5. Self- lubricating ball joints			-	No debris nor free-play			   __________	    __________

6. Star arms end bushes			-	No space between adhesive bead and bush	   __________	    __________

7. Vibration absorber			-	Security					   __________	    __________

8. Blades					-	Security, general coating, tabs, and polyurethane											protection condition (visual check for debonding,											scratches, cracks, impacts and distortions). No											erosion holes on leading edge steel strip, no gaps											nor impacts				   __________	    __________

9. Right cargo door				-	Opening, condition, attachment points, no 											abnormally freeplay.			   __________	    __________

10. Right cargo door			-	Closed and secured				   __________	    __________

11. GPU plug panel				-	Closed or plugged-in, as applicable		   __________	    __________

12. RH MGB cowling			-	Opening, condition of locking systems, no											abnormal freeplay				   __________	    __________

13. Transmission deck			-	Cleanliness				   __________	    __________

14. MGB support bars			-	Condition, security				   __________	    __________

15. Oil cooler, fan and pipes			-	Condition, no leak, fan security, fan blades											condition					   __________	    __________

16. Servos				-	Security, check for leaks or cracks		   __________	    __________

17. Hydraulic system			-	Security, pipes condition, check for leaks, filter											clogging indicator retracted			   __________	    __________

18. Hydraulic system tank			-	Level, no leak				   __________	    __________

19. Engine oil tank				-	Oil level, pipes condition, no leak		   __________	    __________

20. Electrical harnesses			-	Condition, security				   __________	    __________

21. Gimbal ring assembly			-	Fitting, safety pins set and locked		   __________	    __________

22. RH side MGB cowling			-	Closed and secured				   __________	    __________

23. Landing gear				-	 Condition of crosstubes, skids, wear resistant											plates, footstep security			   __________	    __________

24. All lower central fairings			-	Closed and secured				   __________	    __________

25. RH cabin access doors			-	Condition, security, locking, no abnormal freeplay	   __________	    __________

26. Front door jettison system			-	Condition, no crack				   __________	    __________

CABIN INTERIOR

1. Cabin					-	General cleanliness				   __________	    __________

2. Seats					-	Condition, attachment points			   __________	    __________

3. Door jettison system			-	Checked – Plastic guard condition		   __________	    __________

4. Fire extinguisher				-	Secured – Checked				   __________	    __________

5. Circuit breakers				-	All set					   __________	    __________

6. SCU					-	Check all pushbuttons in OFF position.		   __________	    __________

7. Battery switch				-	ON, check battery voltage			   __________	    __________

8. VEMD					-	Check flights of the day report pages data (MAIN											 mode, FLIGHT REPORT page)		   __________	    __________

- VEMD flight times,

- Ng and Nf cycles: check written in white characters and above 0,				   __________	    __________

- Check advisory messages of FAILURE or OVERLIMIT DETECTED,				   __________	    __________

- Record flights of the day data in aircraft and engine logbooks.				   __________	    __________

9. Battery Switch				-	OFF					   __________	    __________

Released by:

____________________________________						        ____________________________________

Mechanic										        Dated

____________________________________

A&P License Nr.